from pydub import AudioSegment

import numpy as np

import argparse

TARGET_SR = 32000

DAC_BITS = 12

DAC_MAX = (1 << DAC_BITS) - 1

DAC_MID = DAC_MAX // 2

DOWNSAMPLE_FACTOR = 1  # в столько раз меньше элементов

def mp3_to_pcm(mp3_path, seconds):

audio = AudioSegment.from_mp3(mp3_path)

audio = audio.set_channels(1)

audio = audio.set_frame_rate(TARGET_SR)

audio = audio[:seconds * 1000]

samples = np.array(audio.get_array_of_samples(), dtype=np.float32)

# нормализация (-1..1)

samples /= np.max(np.abs(samples)) + 1e-9

# уменьшаем массив

samples = samples[::DOWNSAMPLE_FACTOR]

# перевод в uint16 под 12 бит DAC

pcm = (samples * DAC_MID + DAC_MID).astype(np.uint16)

return pcm

def generate_c_header(pcm, var_name):

print("#pragma once")

print("#include <stdint.h>")

print()

print(f"#define {var_name.upper()}_LEN {len(pcm)}")

print(f"#define {var_name.upper()}_DOWNSAMPLE {DOWNSAMPLE_FACTOR}")

print()

print(f"const uint16_t {var_name}[{var_name.upper()}_LEN] = {{")

for i in range(0, len(pcm), 16):

chunk = pcm[i:i+16]

line = ", ".join(f"0x{v:04X}" for v in chunk)

print(f"    {line},")

print("};")

def main():

parser = argparse.ArgumentParser()

parser.add_argument("mp3", help="MP3 file")

parser.add_argument("--seconds", type=int, required=True, help="Duration in seconds")

parser.add_argument("--name", default="music", help="Array name")

args = parser.parse_args()

pcm = mp3_to_pcm(args.mp3, args.seconds)

generate_c_header(pcm, args.name)

if __name__ == "__main__":

main()

python mp3_to_uint16.py music.mp3 --seconds 14 --name music > music.h

from pydub import AudioSegment

import numpy as np

import argparse

TARGET_SR = 32000

DOWNSAMPLE_FACTOR = 1  # в столько раз меньше элементов

def mp3_to_pcm(mp3_path, seconds):

audio = AudioSegment.from_mp3(mp3_path)

audio = audio.set_channels(1)

audio = audio.set_frame_rate(TARGET_SR)

audio = audio[:seconds * 1000]

samples = np.array(audio.get_array_of_samples(), dtype=np.float32)

# нормализация (-1..1)

samples /= np.max(np.abs(samples)) + 1e-9

# уменьшаем массив

samples = samples[::DOWNSAMPLE_FACTOR]

# перевод в uint8 (0..255)

pcm8 = ((samples + 1.0) * 127.5).astype(np.uint8)

return pcm8

def generate_c_header(pcm, var_name):

print("#pragma once")

print("#include <stdint.h>")

print()

print(f"#define {var_name.upper()}_LEN {len(pcm)}")

print(f"#define {var_name.upper()}_DOWNSAMPLE {DOWNSAMPLE_FACTOR}")

print()

print(f"const uint8_t {var_name}[{var_name.upper()}_LEN] = {{")

for i in range(0, len(pcm), 16):

chunk = pcm[i:i+16]

line = ", ".join(f"0x{v:02X}" for v in chunk)

print(f"    {line},")

print("};")

def main():

parser = argparse.ArgumentParser()

parser.add_argument("mp3", help="MP3 file")

parser.add_argument("--seconds", type=int, required=True, help="Duration in seconds")

parser.add_argument("--name", default="music", help="Array name")

args = parser.parse_args()

pcm = mp3_to_pcm(args.mp3, args.seconds)

generate_c_header(pcm, args.name)

if __name__ == "__main__":

main()

python mp3_to_uint8.py music.mp3 --seconds 14 --name music > music.h

from pydub import AudioSegment

import numpy as np

import scipy.signal

import argparse

TARGET_SR = 32000        # частота дискретизации

TIME_STRETCH = 1.07

# Таблица изменения step-index (IMA ADPCM)

INDEX_TABLE = np.array([

-1, -1, -1, -1, 2, 4, 6, 8,

-1, -1, -1, -1, 2, 4, 6, 8

], dtype=np.int32)

# Таблица шагов квантования (IMA ADPCM)

STEP_TABLE = np.array([

7, 8, 9, 10, 11, 12, 13, 14, 16, 17,

19, 21, 23, 25, 28, 31, 34, 37, 41, 45,

50, 55, 60, 66, 73, 80, 88, 97, 107, 118,

130, 143, 157, 173, 190, 209, 230, 253, 279, 307,

337, 371, 408, 449, 494, 544, 598, 658, 724, 796,

876, 963, 1060, 1166, 1282, 1411, 1552, 1707, 1878, 2066,

2272, 2499, 2749, 3024, 3327, 3660, 4026, 4428, 4871, 5358,

5894, 6484, 7132, 7845, 8630, 9493, 10442, 11487, 12635, 13899,

15289, 16818, 18500, 20350, 22385, 24623, 27086, 29794, 32767

], dtype=np.int32)

def time_stretch(samples: np.ndarray, rate: float) -> np.ndarray:

"""Ускоряем или замедляем сигнал без изменения тональности"""

n_samples = int(len(samples) / rate)

stretched = scipy.signal.resample(samples, n_samples)

stretched = np.clip(stretched, -32768, 32767)

return stretched.astype(np.int16)

def ima_adpcm_encode(pcm16: np.ndarray):

"""

Кодируем int16 PCM в IMA ADPCM (4-bit).

Возвращает bytes (packed nibbles) и начальное состояние (predictor + index).

"""

predictor = int(pcm16[0])

index = 0

step = int(STEP_TABLE[index])

adpcm_nibbles = []

for sample in pcm16[1:]:

sample = int(sample)

diff = sample - predictor

code = 0

if diff < 0:

code = 8

diff = -diff

temp_step = step

delta = 0

if diff >= temp_step:

code |= 4

diff -= temp_step

delta += temp_step

temp_step >>= 1

if diff >= temp_step:

code |= 2

diff -= temp_step

delta += temp_step

temp_step >>= 1

if diff >= temp_step:

code |= 1

delta += temp_step

delta += step >> 3

if code & 8:

predictor -= delta

else:

predictor += delta

# Ограничение

predictor = max(-32768, min(32767, predictor))

index += int(INDEX_TABLE[code])

index = max(0, min(88, index))

step = int(STEP_TABLE[index])

adpcm_nibbles.append(code & 0x0F)

# Упакуем 2 nibble в 1 byte

packed = bytearray()

for i in range(0, len(adpcm_nibbles), 2):

if i + 1 < len(adpcm_nibbles):

b = (adpcm_nibbles[i] & 0x0F) | ((adpcm_nibbles[i + 1] & 0x0F) << 4)

else:

b = adpcm_nibbles[i] & 0x0F

packed.append(b)

return packed, pcm16[0], index

def mp3_to_adpcm(mp3_path, seconds):

audio = AudioSegment.from_mp3(mp3_path)

audio = audio.set_channels(1)

audio = audio.set_frame_rate(TARGET_SR)

audio = audio[:seconds * 1000]

samples = np.array(audio.get_array_of_samples(), dtype=np.int16)

# ускоряем без изменения тональности

stretched = time_stretch(samples, TIME_STRETCH)

packed, predictor, index = ima_adpcm_encode(stretched)

return packed, predictor, index

def generate_c_header(adpcm_bytes, var_name, first_sample, index):

print("#pragma once")

print("#include <stdint.h>\n")

print(f"#define {var_name.upper()}_LEN {len(adpcm_bytes)}")

print(f"#define {var_name.upper()}_SR {TARGET_SR}")

print(f"#define {var_name.upper()}_FIRST {int(first_sample)}")

print(f"#define {var_name.upper()}_INDEX {int(index)}\n")

print(f"const uint8_t {var_name}[{var_name.upper()}_LEN] = {{")

for i in range(0, len(adpcm_bytes), 32):

chunk = adpcm_bytes[i:i+32]

line = ", ".join(str(v) for v in chunk)

print(f"    {line},")

print("};")

def main():

parser = argparse.ArgumentParser()

parser.add_argument("mp3", help="MP3 file")

parser.add_argument("--seconds", type=int, required=True, help="Duration in seconds")

parser.add_argument("--name", default="music", help="Array name")

args = parser.parse_args()

adpcm_bytes, first_sample, index = mp3_to_adpcm(args.mp3, args.seconds)

generate_c_header(adpcm_bytes, args.name, first_sample, index)

if __name__ == "__main__":

main()

python mp3_to_uint8.py music.mp3 --seconds 14 --name music > music.h

from pydub import AudioSegment

import numpy as np

import argparse

TARGET_SR = 32000  # частота дискретизации

INDEX_TABLE = np.array([

-1, -1, -1, -1, 2, 4, 6, 8,

-1, -1, -1, -1, 2, 4, 6, 8

], dtype=np.int32)

STEP_TABLE = np.array([

7, 8, 9, 10, 11, 12, 13, 14, 16, 17,

19, 21, 23, 25, 28, 31, 34, 37, 41, 45,

50, 55, 60, 66, 73, 80, 88, 97, 107, 118,

130, 143, 157, 173, 190, 209, 230, 253, 279, 307,

337, 371, 408, 449, 494, 544, 598, 658, 724, 796,

876, 963, 1060, 1166, 1282, 1411, 1552, 1707, 1878, 2066,

2272, 2499, 2749, 3024, 3327, 3660, 4026, 4428, 4871, 5358,

5894, 6484, 7132, 7845, 8630, 9493, 10442, 11487, 12635, 13899,

15289, 16818, 18500, 20350, 22385, 24623, 27086, 29794, 32767

], dtype=np.int32)

def mp3_to_pcm(mp3_path: str, seconds=None):

audio = AudioSegment.from_mp3(mp3_path)

# mono

audio = audio.set_channels(1)

# ставим частоту дискретизации (это единственный ресемплинг, который нужен)

audio = audio.set_frame_rate(TARGET_SR)

# обрезка по времени

if seconds is not None:

audio = audio[:seconds * 1000]

samples = np.array(audio.get_array_of_samples(), dtype=np.int16)

return samples

def ima_adpcm_encode(pcm16: np.ndarray):

"""

PCM int16 -> IMA ADPCM 4-bit

Возвращает:

packed_bytes, first_sample, start_index

"""

if len(pcm16) < 2:

raise ValueError("PCM слишком короткий")

predictor = int(pcm16[0])

index = 0

step = int(STEP_TABLE[index])

adpcm_nibbles = []

for sample in pcm16[1:]:

sample = int(sample)

diff = sample - predictor

code = 0

if diff < 0:

code = 8

diff = -diff

delta = step >> 3

if diff >= step:

code |= 4

diff -= step

delta += step

if diff >= (step >> 1):

code |= 2

diff -= (step >> 1)

delta += (step >> 1)

if diff >= (step >> 2):

code |= 1

delta += (step >> 2)

if code & 8:

predictor -= delta

else:

predictor += delta

predictor = max(-32768, min(32767, predictor))

index += int(INDEX_TABLE[code & 0x0F])

index = max(0, min(88, index))

step = int(STEP_TABLE[index])

adpcm_nibbles.append(code & 0x0F)

packed = bytearray()

for i in range(0, len(adpcm_nibbles), 2):

if i + 1 < len(adpcm_nibbles):

packed.append((adpcm_nibbles[i] & 0x0F) | ((adpcm_nibbles[i + 1] & 0x0F) << 4))

else:

packed.append(adpcm_nibbles[i] & 0x0F)

return packed, int(pcm16[0]), 0

def write_header_file(filename, var_name, adpcm_bytes, first_sample, index):

var_upper = var_name.upper()

with open(filename, "w", encoding="utf-8") as f:

f.write("#pragma once\n")

f.write("#include <stdint.h>\n\n")

f.write(f"#define {var_upper}_LEN {len(adpcm_bytes)}\n")

f.write(f"#define {var_upper}_SR {TARGET_SR}\n")

f.write(f"#define {var_upper}_FIRST {first_sample}\n")

f.write(f"#define {var_upper}_INDEX {index}\n\n")

f.write(f"const uint8_t {var_name}[{var_upper}_LEN] = {{\n")

for i in range(0, len(adpcm_bytes), 32):

chunk = adpcm_bytes[i:i + 32]

line = ", ".join(str(b) for b in chunk)

f.write(f"    {line},\n")

f.write("};\n")

def main():

parser = argparse.ArgumentParser()

parser.add_argument("mp3", help="Путь к mp3 файлу")

parser.add_argument("--seconds", type=int, default=None, help="Обрезка по времени (сек)")

parser.add_argument("--name", default="what", help="Имя массива в C")

parser.add_argument("--out", default="music.h", help="Имя выходного .h файла")

args = parser.parse_args()

pcm = mp3_to_pcm(args.mp3, args.seconds)

duration_sec = len(pcm) / TARGET_SR

print("====== INFO ======")

print("File:", args.mp3)

print("Sample rate:", TARGET_SR)

print("Samples:", len(pcm))

print("Duration:", round(duration_sec, 3), "sec")

print("==================")

adpcm_bytes, first_sample, index = ima_adpcm_encode(pcm)

write_header_file(args.out, args.name, adpcm_bytes, first_sample, index)

print(f"\nГотово! Сохранено в файл: {args.out}")

print(f"ADPCM bytes: {len(adpcm_bytes)}")

print(f"FIRST: {first_sample}")

print(f"INDEX: {index}")

if __name__ == "__main__":

main()

python convert_mp3_to_adpcm.py song.mp3 --seconds 15 --name jump --out jump.h